Chapter 23 (5) Alternating Current In Pure Capacitor

- A sinusoidal voltage of 240 Vrms with frequency 50 Hz is connected to a capacitor of capacitance 10 F. Determine,

- The capacitive reactance of the capacitor.

- The rms current flowing in the circuit.

Ans :  a) 318          b) 0.755 A

- A capacitor of 15 F is connected to a sinusoidal voltage of 200 Vrms and frequency 50 Hz. Determine,

- The maximum voltage that can be applied across the capacitor,

- The capacitive reactance of the capacitor,

- The rms current flowing in the circuit.

Ans :  a) 283 V        b) 212         c) 0.943 A

- A voltage V represented by the equation:

V = (15 V)sin(377t)

is applied across a pure capacitor of capacitance 250 F. Determine,

- The rms voltage,

- The frequency of the voltage,

- The capacitive reactance of the capacitor,

- The rms current flowing through the capacitor,

- The equation representing the current,

- The maximum instantaneous power that could possibly be delivered to the capacitor.

Ans :  a) 10.6 V        b) 60 Hz        c) 10.6         d) 1 A         e) I = 2cos(377t)        f) 10.6 W