Chapter 23 (3) Alternating Current In Pure Resistor

- A coil of area 2.0 cm2 has 20 turns. It is placed in a uniform magnetic field of flux density 50 mT. It then rotates about an axis which passes through its centre and perpendicular to the field at frequency 5 revolution per second. Determine,

- The maximum voltage produced across the coil.

- The rms value of the voltage across the coil.

Ans : a) 6.28 X 10-3 V          b) 4.44 X 10-3 V

- A sinusoidal voltage supply is connected to a resistor. A voltmeter records a reading of 5.5 V across the resistor. The deflection of an ammeter pointer indicates that there is a current of 2.3 A flowing through the resistor. Both instrument record rms values. Determine,

- The maximum voltage that can be applied across the resistor.

- The maximum current that can flow through the resistor.

- The average electrical power supplied to the resistor.

- The resistance of the resistor.

Ans :  a) 7.78 V       b) 3.25 A         c) 12.65 W         d) 2.39 

- A voltage V represented bu equation:

V = (15 V)cos(314t)

is applied across a pure resistor of resistance 20 . Determine,

- The rms voltage across the resistor.

- The average electrical power delivered to the resistor.

- The amount of heat produced by the resistor in 2 minutes.

- The maximum instantaneous power that could possibly be delivered to the resistor.

- The equation representing the current.

Ans :  a) 10.61 V          b) 5.63 W          c) 675.6 J          d) 11.25 W         e) I = (0.75 A)cos(314t)